| ERGONOMICS ANALYSIS – GENERAL |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 – PROVIDE BACKGROUND |  |  |  |  |
| Job/Task: Hydrant Operations | Date: 09/09/24 |  | Analyzed by: Troy T Walsh, LMCIT |  |
| Area/Dept/Location: City of Victoria | Project Number: N/A |  | Employee Name: N/A |  |
| Supervisor: Brady Lee, PW Director | Workstation: Utilities – Water System |  | Job Title/Task: Flushing Operations |  |
| 2 – CURRENT MAJOR STEPS |  | 3 – ERGONOMICS ISSUES |  | 4 – SOLUTIONS |
| Step 1: Manually removing cap from fire hydrant |  | Use of force when caps are stuck or hard to open. Many caps have never been opened, or maintained from original installation. |  | Utilzing mechanical advantage for cap removal. |
| Step 2: Open the fire hydrant to first flush the debris or sediment from the hydrant & watermain stem. |  | The stance of the employee and force needed to manually open a fire hydrant. The pressure on the hand, arm, and body. This could be repeated up to 40 times per day during flushing operations. |  | Mechanical advantage to open/closing the hydrant is completed with a tool. |
| Step 3: Close the hydrant after flushing |  | The stance of the employee and force needed to manually open a fire hydrant. The pressure on the hand, arm, and body. This could be repeated up to 40 times per day during flushing operations. |  | Mechanical advantage to open/closing the hydrant is completed with a tool. |
| Step 4: Connect a diffusion device to the hydrant for flushing operations. |  | There are different devives for different needs/applications. Manual moving of the hose and repetitive motion of the threded connection. |  | Storz connection or quick connection for fire hydrant. Storz connections are locking quarter turn vs. a threaded multiple turn connection |
| Step 5: Open Hydrant for flushing. This could be up to 16-20 rotations for each hydrant. |  | Typical 6” Watermain Hydrant stem is 16-20 rotations, and larger hydrants could be additional rotations. This could add additional body muscle stress when the hydrant is not maintained or is difficult to operate. |  | Mechanical advantage to open hydrant for flushing. This could be up to 16-20 rotations for each hydrant . |
| Step 6: Complete hydrant flushing |  | N/A |  | N/A |
| Step 7: Once completed close the fire hydrant. This could be up to 16-20 rotations for each hydrant. Step 8: Once the hydrant has properly drained, replace the hydrant cap and keep hand tight. |  | Typical 6” Watermain Hydrant stem is 16-20 rotations, and larger hydrants could be additional rotations. This could add additional body muscle stress when the hydrant is not maintained or is difficult to operate. N/A |  | Mechanical advantage to close hydrant after flushing. This could be up to 16-20 rotations for each hydrant N/A |

| 5 - FOLLOW UP |  |  |
| --- | --- | --- |